System Manual

IoT-Based Smart Fruit Dehydrator with Multi-Fruit Presets and Automated Thermal Regulation

Prepared by:

Ciriaco, John Rodolf

Gregorio, Jahcy G.

Selda, Angelie A.

Tangkiyen, Almira Danica S.

May 2026

1. Introduction

The IoT-Based Smart Fruit Dehydrator is a specialized food prevention system engineered to mitigate post-harvest losses in agricultural settings. Traditional sun drying methods are often inconsistent, unhygienic, and weather dependent. This system provides a controlled environment that utilize a vertical flow dehydration mechanism to remove moisture from fruit efficiently. By integrating the ESP32 microcontroller, the system transitions from simple heating box to an intelligent IoT device. It allows for precise thermal regulation, pre-programmed logic for specific fruit types.

2. Purpose of the System

The system is designed to achieve the following objectives:

Hygienic Preservation: to provide an enclosed environment protected from dust, insects, and contaminants.

Nutrient retention: to maintain a steady, low temperature heat flow that dries fruit without cooking it, preserving vitamins and flavor.

Operational Efficiency: to automate the drying cycle, removing the need for manual hourly checks.

Remote Accessibility: to provide users with the freedom to monitor the system status from a distance via mobile application.

3. System Architecture and Components

3.1. Control and connectivity

ESP32 Microcontroller: a powerful 32-bit MCU with integrated Wi-Fi and Bluetooth. It manages the PID-like logic for temperature control and handles all I/O operations.

3.2. Sensing and Monitoring

DHT21 Sensor: a digital sensor used to capture internal climate data. It is higher accuracy and wider range compared to the standard DHT11.

20x4 I2C LCD: a four-line display that provides local feedback, including the current fruit mode, temperature, humidity, and time remaining.

3.3. Thermal and Airflow System

12v PTC Ceramic Heaters (200W): these self-regulating heating elements provide sadfe, consistent warmth.

High-Speed Circulatory Fan: a DC powered fan that creates forced convection, pushing hot air through the trays to ensure the fruit at the bottom dries at the same rate as the fruit at the top.

Relay Module: acts as the high-current switch that the ESP32 uses the toggle the heaters.

3.4. Structural Design

G.I. Chassis: the body is constructed from 16x16x20 inch Galvanized Iron. This material is chosen for its durability, corrosion resistance, and ability to reflect heat internally.

Electronics Compartment: a dedicated, ventilated section of the chassis that protects the ESP32 and wiring from the high heat of the drying chamber.

4. Operating Procedures

4.1. Pre-Operation Check

Ensures the 12V 30A power supply is securely connected to a stable power outlet.

Ensure the ESP32 Microcontroller is plugged into its dedicated power adapter/charger. This initializes the Wi-Fi module, the LCD interface, and the sensor logic.

Check that the G.I. chassis door is closed and the fruit rays are properly aligned

Ensure the passive air vents are not obstructed to allow moisture to escape.

4.2. Starting the System

The system features are secure startup sequence to prevent accidental activation

Power On: plug on the main power. The 20x4 LCD will illuminate and display the welcome message “Welcome to Smart Dehydrator”.

System Unlock: the interface is locked by default for safety. To proceed, Press the Red Button. This will unlock the controls and take you to Mode Selection Screen

Mode Selection:

Manual Mode (Blue Button): Press the Blue Button to enter custom settings. Once in Manual Mode, the 5 yellow Buttons change their function to act as adjustment keys. Use the designated Yellow Buttons to increase or decrease the Temperature and the Time. Confirm settings to begin the dehydration process.

Button 1: Temperature -

Button 2: Temperature +

Button 3: Hours -

Button 4: Hours +

Button 5: Mode: Minutes / Hours

Preset Mode (Default):

If you do not press the blue button, the system remains in Preset Mode. The 5 Yellow Buttons represent pre-programmed fruit profiles:

Button 1: Mango

Button 2: Apple

Button 3: Banana

Button 4: Orange

Button 5: Grapes

After pressing your chosen fruit button, you must press the Blue Button to confirm your selection. The drying cycle will only begin once the Blue Button is clicked.

4.3. During Operation

Live Monitoring: The LCD  will display the current internal temperature and remaining time.

Thermal Regulation: The ESP32 will automatically toggle the heaters to maintain the target temperature while the fan ensures even heat distribution.

4.4. System Shutdown and Emergency Stop

Manual Stop: Press the Red Button at any time to immediately kill power to the heaters and fans.

Automatic Completion: When the timer reaches zero, the system will display "Drying Complete" and shut down all heating components.

5. Safety Precautions

Electrical Safety: do not open the electronics compartment while the system is powered.

Heat Safety: the G.I. exterior can become warm during long cycles; handle with care.

Operational Safety: do not overload trays with excessively thick fruit slices, as this may impede airflow and cause uneven drying.

Emergency Stop: if any burning smell is detected, immediately unplug the 12V power source.

6. Maintenance and Cleaning

Trays: wash with mild soap and dry completely before re-insertion

Sensor Care: periodically wipe the DHT22 sensor housing with dry cloth to remove dust buildup that might affect accuracy.

Chassis: Clean the interior G.I. walls with damp cloth to remove ant fruit sugars or residue.

7. Troubleshooting Guide

| Issue | Possible Cause | Corrective Action |
| --- | --- | --- |
| LCD is blank | Power supply failure or loose I2C wires. | Check the 12V input and the 4-pin connector on the LCD. |
| Temperature won't rise | Relay failure or heater burnout. | Check if the relay LED is triggering; replace heater if necessary. |
| Humidity stays high | Fruit is too crowded or vents are blocked. | Reduce the amount of fruit per tray and check air vents. |
| Inaccurate Readings | DHT22 sensor is faulty. | Replace the sensor module. |

8. Advantages and Limitations

Advantages

Precision: Eliminates the guesswork associated with traditional drying

User-Friedly: The 7-button matrix makes it easy for non-technical users to operate

Scalable:  The software can be updated to include more fruit types or drying profiles

Safe: Low-voltage DC components significantly reduce the risk of fire.

Limitations

Batch Size: Limited by the physical dimensions of the 16x16x20 inch chassis.

Power Supply: Requires a high-amperage 12V source, making it dependent on grid power or large batteries.

9. Conclusion

The IoT-Based Smart Fruit Dehydrator is a comprehensive solution for modern food preservation. It ensures that fruit preservation is no longer a game of chance but a controlled, data driven process. With proper maintenance and operation, this system serves as a vital tool for reducing waste and improving food security.